ASSIGNMENT-2

School of Engineering and Technology
Course Code & Name: ENCS304 Computer Networks

Program Name: B.Tech CSE

Name: N. Yugesh

Roll Number: 2301010199

Course: BTech CSE – C

Packet Flow Visualization Using Simulation Mode

Experiment Objectives:

Students will use Packet Tracer Simulation Mode to observe how packets move between devices, how switches learn MAC addresses, and how ICMP echo request/reply travels through a small network.

Learning Outcomes

- Use Simulation Mode effectively

- Observe ARP + ICMP behavior

- Understand switch MAC learning behavior

- Interpret packet event list timeline

Concepts Used

- Simulation Mode

- ICMP ping

- ARP resolution

- Switch MAC address table learning

Objective

The objective of this experiment is to understand how packets travel across a network using Simulation Mode in Cisco Packet Tracer. It also demonstrates ARP resolution, ICMP communication, and how switches learn MAC addresses.

Task 1: Building a Simple LAN

A simple Local Area Network (LAN) was created using:

- 1 Switch

- 3 PCs

IP Configuration

- PC1 → 192.168.20.1 /24

- PC2 → 192.168.20.2 /24

- PC3 → 192.168.20.3 /24

All devices were connected to the switch using straight-through cables.

Task 2: First Ping Observation (PC1 → PC2)

Simulation Mode was enabled and a ping was sent from PC1 to PC2.

Observed Behavior

- ARP Request

  - PC1 did not know the MAC address of PC2.

  - It sent an ARP Request (broadcast) to all devices in the network.

  - The switch forwarded this broadcast frame to all ports except the source port.

- ARP Reply

  - PC2 responded with its MAC address using an ARP Reply (unicast) to PC1.

  - The switch forwarded this reply directly to PC1.

- ICMP Echo Request

  - After ARP resolution, PC1 sent an ICMP Echo Request (ping) to PC2.

- ICMP Echo Reply

  - PC2 responded with an ICMP Echo Reply.

  - Communication was successfully completed.

Switch Behavior

- The switch learned MAC addresses by observing incoming frames.

- It stored:

  - PC1 MAC → Port X

  - PC2 MAC → Port Y

Task 3: Second Ping Observation (PC1 → PC2)

A second ping was sent from PC1 to PC2.

Observed Behavior

- No ARP request was generated.

- Only:

  - ICMP Echo Request

  - ICMP Echo Reply

Reason

- PC1 already had PC2’s MAC address stored in its ARP cache.

- The switch also had MAC entries in its MAC address table.

Conclusion

- The second ping was faster and involved fewer packets.

Task 4: MAC Address Table Observation

The switch MAC address table was checked (or inferred through simulation events).

Observations

- The switch dynamically learned MAC addresses of connected PCs.

- Entries included:

  - MAC address of PC1 mapped to its port

  - MAC address of PC2 mapped to its port

Learning Mechanism

- When a frame arrives, the switch records:

  - Source MAC address

  - Incoming port

Screenshots

Setup

Ping

transmission chronology

Packet received

Sending packet again after caching

Packet received

Switch table (after caching)